GEOMETRY OF POINTS, RAYS, PLANES AND CYLINDERS

# Ray

{r}

{
}

{x}

s

{x} = general location along ray

{r} = known location on ray

{u} = unit direction of ray

s = directed distance along ray from {r} to {x}

{x} = {r} + s {u}

X

{
}

{x}

{p}

Y



EDGE VIEW

OF PLANE

# Plane

{x} = general location on plane

{p} = known location on plane

{n} = unit normal to plane

 = directed distance along perpendicular from origin to plane

{n}T {x} = {n}T {p} = 

{
}

{p}

{x}



# Cylinder

{x} = general location on cylinder

{p} = location on axis of cylinder

{n} = unit direction along axis of cylinder

= radius of cylinder

({x}-{p})T [n]2 ({x}-{p}) = 

{n}=nxnynz          {n}T{n} = 1

[n] = 0−nznynz0−nx−nynx0          [n]T =  -[n]

[n]2 = nx2−1nxnynxnznxnyny2−1nynznxnznynznz2−1= {n}{n}T-[ I3 ]       symmetric idempotent [n]2 [n]2 = [n]2

# Perpendicular from Point to Ray

given	{q} = known location of point

{r} = known location on ray

{u} = known unit direction along ray

{r}

{
}

{q}

s

{d}

[u] = skew-symmetric matrix for {u}

find	s = directed distance along {u} from {r} to root of perpendicular

s = {u}T ({q}-{r})

find	{d} = vector from {q} to root of perpendicular

{d} = {u} x {u} x ({q}-{r}) = [u]2 ({q}-{r})

# Perpendicular from Point to Plane

given	{q} = location of point

{p} = known location on plane

{n} = known unit normal to plane

find	d  = directed distance along {n} from {q} to root of perpendicular

d = {n}T({p}-{q})

{q}

{
}

{p}

{d}

EDGE VIEW

OF PLANE

find	{d} = vector from {q} to root of perpendicular

{d}  =  {n}d  =  {n}{n}T ({p}-{q})

# Intersection of Ray and Plane

given	{r} = known location on ray

{u} = known unit direction of ray

{p} = known location on plane

{r}

{
}

{p}

s

EDGE VIEW

OF PLANE

{
}

{n} = known unit normal to plane

ray   {x} = {r} + s {u}

plane   {n}T {x} = {n}T {p}

find	s = directed distance along {u} from {r} to plane

substitute ray equation into plane equation

{n}T {r} + s {n}T {u} = {n}T {p}

s = {n}T ({p}-{r})  /  ({n}T {u})

# Intersection of Ray and Cylinder

{
}

{p}

{
}

{r}

s

{x1}

{x2}



given	{r} = known location on ray

{u} = known unit direction of ray

{p} = known location on axis of cylinder

{n} = known unit direction along axis of cylinder

 = known radius of cylinder

find	s = directed distance along {u} from {r} to cylinder (two solutions at {x1} and {x2})

{d} = vector from {x} to root of perpendicular with axis of cylinder          {d} = [n]2 ({x}-{p})

{d}T{d} = 2({x}-{p})T [n]2 ({x}-{p})

ray   {x} = {r} + s{u}

2( {r} + s{u}-{p} )T [n]2 ( {r} + s{u}-{p} )

s2{u}T[n]2{u} + 2s({r}-{p})T[n]2{u} + ({r}-{p})T [n]2 ({r}-{p})

a s2 + b s + c  =  0

a  =  {u}T [n]2 {u}

b  =  2 ({r}-{p})T [n]2 {u}

c  =  ({r}-{p})T [n]2 ({r}-{p}) - 

s1 , s2  = −b ± b2 − 4ac2a

if a = 0, ray is parallel to axis of cylinder

if  b2 < 4ac, ray does not intersect cylinder

if  b2 = 4ac, ray is tangent to cylinder

if  s1 and s2 have opposite signs, point {r} is inside cylinder

Common Perpendicular between Two Rays

{r1}

{r2}

{
1}

{
2}

{q}

s2

s1

d

given	{r1} = known location on ray 1

{u1} = known unit direction of ray 1

{r2} = known location on ray 2

{u2} = known unit direction of ray 2

find	s1 = directed distance along {u1} from {r1} to respective root of common perpendicular

s2 = directed distance along {u2} from {r2} to respective root of common perpendicular

 = angle between rays          sin  = norm( {u1} x {u2} ) = norm( [u1] {u2} )

{u3} = unit direction along common perpendicular          {u3} = [u1] {u2} / sin 

d = directed distance (length of common perpendicular) along {u3} from ray 1 to ray 2

{r1} + s1 {u1} + d {u3} = {r2} + s2 {u2}

u1−u2u3s1s2d=r2−r1

s1s2d=u1−u2u3−1 r2−r1

# OR

find {q} = midpoint of common perpendicular using confluence of multiple rays with m=2

{q}  =  ( [u1] 2 + [u2] 2 )-1   (  [u1] 2 {r1} + [u2] 2 {r2} )

Confluence Point of Multiple Rays  (m  2)

given	{ri} = known location on ray i,  i = 1 to m

{ui} = known unit direction of ray i

find	{q} = least-squares confluence of rays

for m = 2,  {q} = midpoint of common perpendicular (least-squares intersection)

{di} = vector from confluence to root of perpendicular to each ray i         {di}  =  [ui] 2 ({q}-{ri})

SSQ  =  Σmi=1{di}T{di}  = Σmi=1 ({q}-{ri}) T[ui] 2 ({q}-{ri})

SSQ  =  {q}T (Σmi=1[ui] 2 ) {q} - 2{q}T Σmi=1 ( [ui] 2 {ri}) + Σmi=1 ({ri} T[ui] 2 {ri})

minimize SSQ with respect to {q}  by setting   SSQ / {q}T  =  0

SSQ / {q}T  =  2 (Σmi=1 [ui] 2 ) {q} - 2 Σmi=1 ( [ui] 2 {ri})  =  0

{q}  =  (Σmi=1 [ui] 2 )-1   Σmi=1 ( [ui] 2 {ri})

{
i}

{ri}

{q}

for  m > 2

for  m = 2

{r1}

{r2}

{
1}

{
2}

{q}

# Intersection of Two Planes

{
1}

{
2}

{p2}

{p1}

{
3}

s

{
}

{q}



given	{p1} = known location on plane 1

{n1} = known unit normal to plane 1

{p2} = known location on plane 2

{n2} = known unit normal to plane 2

find	{q} = location on line of intersection

{u} = unit direction for line of intersection

 = dihedral angle between planes          sin  = norm( {n1} x {n2} ) = norm( [n1] {n2} )

{n3} = unit vector mutually perpendicular to {n1} and {n2}          {n3} =  [n1] {n2} / sin 

{u} = unit vector perpendicular to {n1} and {n3}

{u} = {n1} x {n3} = {n1} x {n1} x {n2} / sin  = [n1] [n1] {n2} / sin 

{q} will lie along {u} from {p1}           {q} = {p1} + s {u}

{q} must lie in plane 2      {n2}T{q} = {n2}T{p2}

{n2}T{p1} + s {n2}T {u} = {n2}T{p2}

{n2}T ({p2}-{p1}) = s {n2}T [n1] [n1] {n2} / sin = -s ({n2}T [n1]T) ([n1] {n2})/ sin 

{n2}T ({p2}-{p1}) = -s (sin {n3}T) (sin {n3}) / sin = -s  sin 

s = - ( {n2}T ({p2}-{p1})  )  / sin 

{q} = {p1} + s {u}

# OR

create a third mutually orthogonal plane 3 with {n3} and {p3}={p1}

then solve for the intersection of these three planes to find {q}

q=n1Tn2Tn3T−1 n1Tp1n2Tp2n3Tp1

Confluence of Multiple Planes (m  3)

{
1}

{
2}

{p2}

{p1}

{
3}

{p3}

{q}

for  m = 3

given	{pi} = known location on plane i,  i =  1 to m

{ni} = known unit normal to plane i

find	{q} = confluence point

{ni}T {q} = {ni}T {pi}   for all i = 1 to m

for m = 3,  {q} = true intersection point

n1Tn2Tn3Tq=n1Tp1n2Tp2n3Tp3

q=n1Tn2Tn3T−1 n1Tp1n2Tp2n3Tp3

for m > 3,  {q} = least-squares confluence point

n1Tn2T⋮nmTq≈n1Tp1n2Tp2⋮nmTpm

{q}  =  (Σmi=1({ni} {ni}T ) )-1   Σmi=1 ({ni} {ni}T {pi})

Confluence of an Axode

For a pencil of n rays respectively passing through points {pi} with unit directions {ui}, the confluence {q} should have the minimum sum of squares SSQr of perpendicular distances from all rays.  Individual perpendicular vectors {ri} from the confluence may be determined using cross product notation or skew-symmetric direction matrices [ui] as shown in Equation D1.  Consequently the sum of squares of perpendicular distances is given in Equation D2 noting that the square of a skew-symmetric direction matrix is idempotent.  Setting the partial derivative of the sum of squares with respect to the confluence equal to zero will minimize the sum of squares as shown in Equation D3.

{ri} = {ui}  {ui}  ({q} - {pi})  =  [ui]2 ({q} - {pi})	(D1)

SSQr =  {ri}T{ri} =   ({q} - {pi})T  [ui]2 ({q} - {pi})       ( for i=1 to n)	(D2)

{q} = ( [ui]2 )-1  ( [ui]2 {pi})	(D3)

The central direction {uo} should have minimum sum of squares SSQ of angles from all rays.  The sine of each angle i can be defined using cross product or skew symmetric matrix notation in Equation D4.  For small angular dispersion (i < 15 degrees) the sum of squares of angles may be approximated using Equation D5.  Setting the partial derivative of the sum of squares with respect to the central direction equal to zero as shown in Equation D6 will minimize the sum of squares by the eigensolution shown in Equation D7.  For very small angular dispersion (i < 5 degrees) the simple approximation in Equation D8 may also be appropriate.

sin i = norm({ui}  {uo})  =  norm([ui]{uo})	(D4)

SSQ =   ({uo}T [ui]2 {uo})	(D5)

([ui]2) {uo} = 0	(D6)

{uo} =  eigenvector of  [ui]2  for smallest eigenvalue	(D7)

{uo}  ({ui}/n) / norm({ui}/n)	(D8)

extensive notes, code and images for 2D and 3D geometry

http://paulbourke.net/geometry/

http://paulbourke.net/geometry/pointlineplane/

http://paulbourke.net/geometry/circlesphere/

http://paulbourke.net/geometry/transformationprojection/

http://paulbourke.net/geometry/ellipsecirc/